DIGITAL SIGNAL PROCESSING LABORATORY

EXPERIMENT NO. 2

Experiment Name: Design and Performance Analysis of Chebyshev Type-I IIR Low-Pass and High-Pass Filters Using MATLAB

Date of Experiment: 12 October 2026

# Objectives

To design Chebyshev Type-I IIR low-pass and high-pass digital filters using MATLAB.

To determine the filter order and passband edge frequency using the cheb1ord function.

To calculate the filter numerator and denominator coefficients using the cheby1 function.

To plot and analyze the magnitude and phase responses of the designed filters.

# Program Code

clear;

clc;

close all;

% Design Parameters

Fs = 10000;         % Sampling frequency in Hz

Ap = 0.5;           % Passband ripple in dB

As = 30;            % Stopband attenuation in dB

% 1. Low-Pass Filter Design

fp_lp = 1500;       % Passband frequency (Hz)

fs_lp = 2000;       % Stopband frequency (Hz)

wp_lp = fp_lp / (Fs/2);

ws_lp = fs_lp / (Fs/2);

[N_lp, Wn_lp] = cheb1ord(wp_lp, ws_lp, Ap, As);

[b_lp, a_lp] = cheby1(N_lp, Ap, Wn_lp, 'low');

% 2. High-Pass Filter Design

fp_hp = 2500;       % Passband frequency (Hz)

fs_hp = 1800;       % Stopband frequency (Hz)

wp_hp = fp_hp / (Fs/2);

ws_hp = fs_hp / (Fs/2);

[N_hp, Wn_hp] = cheb1ord(wp_hp, ws_hp, Ap, As);

[b_hp, a_hp] = cheby1(N_hp, Ap, Wn_hp, 'high');

% Display numerical outputs

fprintf('=== EXPERIMENT 2: CHEBYSHEV TYPE-I IIR FILTERS ===\n\n');

fprintf('--- CHEBYSHEV LOW-PASS FILTER ---\n');

fprintf('Calculated Filter Order (N_lp): %d\n', N_lp);

fprintf('Passband Edge Cutoff Frequency (Wn_lp): %.4f Hz\n', Wn_lp * (Fs/2));

fprintf('Numerator Coefficients (b_lp):\n');

disp(b_lp);

fprintf('Denominator Coefficients (a_lp):\n');

disp(a_lp);

fprintf('--- CHEBYSHEV HIGH-PASS FILTER ---\n');

fprintf('Calculated Filter Order (N_hp): %d\n', N_hp);

fprintf('Passband Edge Cutoff Frequency (Wn_hp): %.4f Hz\n', Wn_hp * (Fs/2));

fprintf('Numerator Coefficients (b_hp):\n');

disp(b_hp);

fprintf('Denominator Coefficients (a_hp):\n');

disp(a_hp);

% Frequency response calculation

[H_lp, f] = freqz(b_lp, a_lp, 1024, Fs);

[H_hp, ~] = freqz(b_hp, a_hp, 1024, Fs);

% Plotting

figure('Name', 'Chebyshev Type-I LPF and HPF Responses');

subplot(2,2,1);

plot(f, 20*log10(abs(H_lp)), 'b', 'LineWidth', 1.5);

grid on;

title(['Chebyshev LPF Magnitude (Order N = ', num2str(N_lp), ')']);

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

ylim([-60 5]);

subplot(2,2,2);

plot(f, angle(H_lp)*180/pi, 'r', 'LineWidth', 1.5);

grid on;

title('Chebyshev LPF Phase Response');

xlabel('Frequency (Hz)');

ylabel('Phase (degrees)');

subplot(2,2,3);

plot(f, 20*log10(abs(H_hp)), 'b', 'LineWidth', 1.5);

grid on;

title(['Chebyshev HPF Magnitude (Order N = ', num2str(N_hp), ')']);

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

ylim([-60 5]);

subplot(2,2,4);

plot(f, angle(H_hp)*180/pi, 'r', 'LineWidth', 1.5);

grid on;

title('Chebyshev HPF Phase Response');

xlabel('Frequency (Hz)');

ylabel('Phase (degrees)');

# MATLAB Output Screenshot

Figure 1: Chebyshev Type-I low-pass and high-pass magnitude and phase responses.

Done by

Lokesh A

Register Number: 192512293

B.E. Electronics and Communication Engineering (ECE)