DIGITAL SIGNAL PROCESSING LABORATORY

EXPERIMENT NO. 6

Design and Analysis of FIR Digital Filters Using Fourier Series and Windowing Techniques Using MATLAB

Date of Experiment: 12 October 2026

# Objectives

To design FIR low-pass digital filters using different windowing techniques.

To compare Rectangular, Hamming, Hanning and Blackman window responses.

To plot and analyze the magnitude response of the designed FIR filters.

To observe the phase response of the Hamming-window FIR filter.

# Program Code

clear;

clc;

close all;

Fs = 8000;

fc = 1500;

wc = fc/(Fs/2);

N = 50;

% Window Functions

w_rect = rectwin(N+1);

w_hamm = hamming(N+1);

w_hann = hann(N+1);

w_black = blackman(N+1);

% FIR Filter Design

b_rect = fir1(N,wc,w_rect);

b_hamm = fir1(N,wc,w_hamm);

b_hann = fir1(N,wc,w_hann);

b_black = fir1(N,wc,w_black);

% Frequency Response

[H_rect,f] = freqz(b_rect,1,1024,Fs);

[H_hamm,~] = freqz(b_hamm,1,1024,Fs);

[H_hann,~] = freqz(b_hann,1,1024,Fs);

[H_black,~] = freqz(b_black,1,1024,Fs);

% Display Coefficients

fprintf('EXPERIMENT 4: FIR WINDOWING TECHNIQUES\\n\\n');

fprintf('Filter Order: %d\\n',N);

fprintf('Sampling Frequency: %d Hz\\n',Fs);

fprintf('Cutoff Frequency: %d Hz\\n',fc);

fprintf('\\nRECTANGULAR WINDOW COEFFICIENTS (First 5)\\n');

disp(b_rect(1:5));

fprintf('HAMMING WINDOW COEFFICIENTS (First 5)\\n');

disp(b_hamm(1:5));

fprintf('HANNING WINDOW COEFFICIENTS (First 5)\\n');

disp(b_hann(1:5));

fprintf('BLACKMAN WINDOW COEFFICIENTS (First 5)\\n');

disp(b_black(1:5));

% Plot Responses

figure('Name','FIR Windowing Techniques Comparison');

subplot(2,1,1);

plot(f,20*log10(max(abs(H_rect),1e-10)),'k','LineWidth',1);

hold on;

plot(f,20*log10(max(abs(H_hamm),1e-10)),'b','LineWidth',1.5);

plot(f,20*log10(max(abs(H_hann),1e-10)),'g','LineWidth',1.5);

plot(f,20*log10(max(abs(H_black),1e-10)),'r','LineWidth',1.5);

grid on;

legend('Rectangular','Hamming','Hanning','Blackman');

title('FIR Low-Pass Filter Magnitude Response (N = 50)');

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

ylim([-100 10]);

xlim([0 Fs/2]);

subplot(2,1,2);

plot(f,angle(H_hamm)*180/pi,'b','LineWidth',1.5);

grid on;

title('Linear Phase Response (Hamming Window Example)');

xlabel('Frequency (Hz)');

ylabel('Phase (degrees)');

xlim([0 Fs/2]);

ylim([-200 200]);

# MATLAB Output Screenshots

Figure 1: FIR low-pass magnitude responses for Rectangular, Hamming, Hanning and Blackman windows, with the Hamming-window phase response.

Figure 2: MATLAB Command Window output showing the design parameters and the first five coefficients for each window.

# Done By

Lokesh A (192512293)

B.E. Electronics and Communication Engineering (ECE)