DIGITAL SIGNAL PROCESSING LABORATORY

EXPERIMENT NO. 4

Experiment Name: Design and Comparative Analysis of Butterworth, Chebyshev Type-I and Elliptic IIR Filters Using MATLAB

Date of Experiment: 12 October 2026

# Objectives

To design Butterworth, Chebyshev Type-I and Elliptic low-pass IIR filters using MATLAB.

To compare the filter orders required to satisfy the given passband-ripple and stopband-attenuation specifications.

To analyze and compare the magnitude and phase responses of the three filters.

To measure passband ripple, stopband attenuation and transition bandwidth.

# Program Code

clc;

clear;

close all;

%% Design and Comparative Analysis of IIR Filters

Fs = 1000;

Fp = 100;

Fst = 150;

Rp = 1;

Rs = 40;

Wp = Fp/(Fs/2);

Wst = Fst/(Fs/2);

%% 1. Butterworth Filter

[Nb, Wb] = buttord(Wp, Wst, Rp, Rs);

[bB, aB] = butter(Nb, Wb, 'low');

%% 2. Chebyshev Type-I Filter

[Nc, Wc] = cheb1ord(Wp, Wst, Rp, Rs);

[bC, aC] = cheby1(Nc, Rp, Wc, 'low');

%% 3. Elliptic Filter

[Ne, We] = ellipord(Wp, Wst, Rp, Rs);

[bE, aE] = ellip(Ne, Rp, Rs, We, 'low');

%% Display Filter Orders

fprintf('\nFILTER ORDER COMPARISON\n');

fprintf('Butterworth       = %d\n', Nb);

fprintf('Chebyshev Type-I  = %d\n', Nc);

fprintf('Elliptic          = %d\n', Ne);

%% Display Coefficients

fprintf('\nBUTTERWORTH COEFFICIENTS\n');

disp('Numerator:');

disp(bB);

disp('Denominator:');

disp(aB);

fprintf('\nCHEBYSHEV TYPE-I COEFFICIENTS\n');

disp('Numerator:');

disp(bC);

disp('Denominator:');

disp(aC);

fprintf('\nELLIPTIC COEFFICIENTS\n');

disp('Numerator:');

disp(bE);

disp('Denominator:');

disp(aE);

%% Frequency Response

Nfft = 4096;

[HB, f] = freqz(bB, aB, Nfft, Fs);

[HC, ~] = freqz(bC, aC, Nfft, Fs);

[HE, ~] = freqz(bE, aE, Nfft, Fs);

MB = 20*log10(max(abs(HB), 1e-12));

MC = 20*log10(max(abs(HC), 1e-12));

ME = 20*log10(max(abs(HE), 1e-12));

%% Performance Analysis

P = f <= Fp;

S = f >= Fst;

RippleB = max(MB(P)) - min(MB(P));

RippleC = max(MC(P)) - min(MC(P));

RippleE = max(ME(P)) - min(ME(P));

StopB = -max(MB(S));

StopC = -max(MC(S));

StopE = -max(ME(S));

fprintf('\nPASSBAND RIPPLE (dB)\n');

fprintf('Butterworth       = %.4f\n', RippleB);

fprintf('Chebyshev Type-I  = %.4f\n', RippleC);

fprintf('Elliptic          = %.4f\n', RippleE);

fprintf('\nSTOPBAND ATTENUATION (dB)\n');

fprintf('Butterworth       = %.4f\n', StopB);

fprintf('Chebyshev Type-I  = %.4f\n', StopC);

fprintf('Elliptic          = %.4f\n', StopE);

fprintf('\nTRANSITION BANDWIDTH = %.2f Hz\n', Fst-Fp);

%% Magnitude Response Comparison

figure('Name', 'IIR Magnitude Response Comparison');

plot(f, MB, 'LineWidth', 1.5);

hold on;

plot(f, MC, 'LineWidth', 1.5);

plot(f, ME, 'LineWidth', 1.5);

xline(Fp, '--', 'Fp');

xline(Fst, '--', 'Fst');

yline(-Rp, ':', 'Passband limit');

yline(-Rs, ':', 'Stopband limit');

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Comparison of IIR Filter Magnitude Responses');

legend('Butterworth', 'Chebyshev Type-I', 'Elliptic', ...

'Location', 'southwest');

xlim([0 Fs/2]);

ylim([-80 5]);

hold off;

%% Phase Response Comparison

PhaseB = unwrap(angle(HB))*180/pi;

PhaseC = unwrap(angle(HC))*180/pi;

PhaseE = unwrap(angle(HE))*180/pi;

figure('Name', 'IIR Phase Response Comparison');

plot(f, PhaseB, 'LineWidth', 1.3);

hold on;

plot(f, PhaseC, 'LineWidth', 1.3);

plot(f, PhaseE, 'LineWidth', 1.3);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (degrees)');

title('Comparison of IIR Filter Phase Responses');

legend('Butterworth', 'Chebyshev Type-I', 'Elliptic');

xlim([0 Fs/2]);

hold off;

%% Filter Order Comparison

figure('Name', 'IIR Filter Order Comparison');

orders = [Nb Nc Ne];

bar(orders);

set(gca, 'XTick', 1:3);

set(gca, 'XTickLabel', {'Butterworth', 'Chebyshev-I', 'Elliptic'});

ylabel('Filter Order');

xlabel('Filter Type');

title('Comparison of IIR Filter Orders');

grid on;

for k = 1:3

text(k, orders(k), sprintf('%d', orders(k)), ...

'HorizontalAlignment', 'center', ...

'VerticalAlignment', 'bottom');

end

# MATLAB Output Screenshots

Figure 1: Comparison of IIR filter magnitude responses, phase responses and filter orders.

Figure 2: MATLAB Command Window showing the calculated filter coefficients and performance measurements.

Done by

Lokesh A

Register Number: 192512293

B.E. Electronics and Communication Engineering (ECE)