DIGITAL SIGNAL PROCESSING LABORATORY

EXPERIMENT NO. 1

Experiment Name: Design, Realization and Performance Analysis of Butterworth IIR Low-Pass and High-Pass Digital Filters Using MATLAB

Date of Experiment: 12 October 2026

# Objectives

To design Butterworth IIR low-pass and high-pass digital filters using MATLAB.

To calculate the filter order and cutoff frequency using the buttord function.

To obtain the numerator and denominator coefficients using the butter function.

To plot and observe the magnitude and phase responses of both filters.

# Program Code

clear;

clc;

close all;

% Design Parameters

Fs = 8000;      % Sampling frequency in Hz

Ap = 1;         % Passband ripple in dB

As = 40;        % Stopband attenuation in dB

% 1. Low-Pass Filter Design

fp_lp = 1000;   % LPF Passband frequency (Hz)

fs_lp = 1500;   % LPF Stopband frequency (Hz)

wp_lp = fp_lp / (Fs/2);

ws_lp = fs_lp / (Fs/2);

[N_lp, Wn_lp] = buttord(wp_lp, ws_lp, Ap, As);

[b_lp, a_lp] = butter(N_lp, Wn_lp, 'low');

% 2. High-Pass Filter Design

fp_hp = 2500;   % HPF Passband frequency (Hz)

fs_hp = 1800;   % HPF Stopband frequency (Hz)

wp_hp = fp_hp / (Fs/2);

ws_hp = fs_hp / (Fs/2);

[N_hp, Wn_hp] = buttord(wp_hp, ws_hp, Ap, As);

[b_hp, a_hp] = butter(N_hp, Wn_hp, 'high');

% Display numerical outputs

fprintf('=== EXPERIMENT 1: BUTTERWORTH IIR FILTERS ===\n\n');

fprintf('--- LOW-PASS FILTER ---\n');

fprintf('Calculated Filter Order (N_lp): %d\n', N_lp);

fprintf('3-dB Cutoff Frequency (Wn_lp): %.4f Hz\n', Wn_lp * (Fs/2));

fprintf('Numerator Coefficients (b_lp):\n');

disp(b_lp);

fprintf('Denominator Coefficients (a_lp):\n');

disp(a_lp);

fprintf('--- HIGH-PASS FILTER ---\n');

fprintf('Calculated Filter Order (N_hp): %d\n', N_hp);

fprintf('3-dB Cutoff Frequency (Wn_hp): %.4f Hz\n', Wn_hp * (Fs/2));

fprintf('Numerator Coefficients (b_hp):\n');

disp(b_hp);

fprintf('Denominator Coefficients (a_hp):\n');

disp(a_hp);

% Frequency response calculation

[H_lp, f] = freqz(b_lp, a_lp, 1024, Fs);

[H_hp, ~] = freqz(b_hp, a_hp, 1024, Fs);

% Plotting

figure('Name', 'Butterworth IIR LPF and HPF');

subplot(2,2,1);

plot(f, 20*log10(abs(H_lp)), 'b', 'LineWidth', 1.5);

grid on;

title(['Butterworth LPF Magnitude (Order N = ', num2str(N_lp), ')']);

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

ylim([-80 5]);

subplot(2,2,2);

plot(f, angle(H_lp)*180/pi, 'r', 'LineWidth', 1.5);

grid on;

title('Butterworth LPF Phase Response');

xlabel('Frequency (Hz)');

ylabel('Phase (degrees)');

subplot(2,2,3);

plot(f, 20*log10(abs(H_hp)), 'b', 'LineWidth', 1.5);

grid on;

title(['Butterworth HPF Magnitude (Order N = ', num2str(N_hp), ')']);

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

ylim([-80 5]);

subplot(2,2,4);

plot(f, angle(H_hp)*180/pi, 'r', 'LineWidth', 1.5);

grid on;

title('Butterworth HPF Phase Response');

xlabel('Frequency (Hz)');

ylabel('Phase (degrees)');

# MATLAB Output Screenshots

Figure 1: Butterworth IIR low-pass and high-pass magnitude and phase responses.

Figure 2: MATLAB Online workspace showing the program and generated filter plots.

Done by

Lokesh A

Register Number: 192512293

B.E. Electronics and Communication Engineering (ECE)