DIGITAL SIGNAL PROCESSING LABORATORY

EXPERIMENT NO. 5

Realization of IIR Filters Using Direct Form-I, Direct Form-II, Cascade and Parallel Structures Using MATLAB

Date of Experiment: 12 October 2026

# Objectives

To realize an IIR digital filter using Direct Form-I and Direct Form-II structures.

To obtain cascade realization using second-order sections.

To obtain parallel realization using partial-fraction expansion.

To compare impulse responses and numerical errors of the different realizations.

To plot the frequency response and pole-zero plot of the IIR filter.

# Program Code

clc;

clear;

close all;

format short;

%% IIR Filter Coefficients

b = [0.0048 0.0193 0.0289 0.0193 0.0048];

a = [1 -2.3695 2.3140 -1.0547 0.1874];

N = 50;

x = [1 zeros(1,N-1)];  % Unit impulse

n = 0:N-1;

%% 1. Direct Form-I and Direct Form-II

y1 = filter(b,a,x);

y2 = filter(b,a,x);

%% 2. Cascade Realization

[sos,g] = tf2sos(b,a);

yc = g*x;

for k = 1:size(sos,1)

yc = filter(sos(k,1:3),sos(k,4:6),yc);

end

%% 3. Parallel Realization

[r,p,k] = residuez(b,a);

yp = filter(k,1,x);

for i = 1:length(r)

yp = yp + filter(r(i),[1 -p(i)],x);

end

yp = real(yp);

%% 4. Display Coefficients

disp('Numerator coefficients b ='); disp(b);

disp('Denominator coefficients a ='); disp(a);

disp('Second Order Sections (Cascade) ='); disp(sos);

disp('Cascade Gain ='); disp(g);

disp('Residues (Parallel) ='); disp(r);

disp('Poles (Parallel) ='); disp(p);

disp('Direct term ='); disp(k);

%% 5. Error and Percentage Deviation

E = [max(abs(y1-y2)); ...

max(abs(y1-yc)); ...

max(abs(y1-yp))];

D = E/max(abs(y1))*100;

Structure = {'DF-I'; 'DF-II'; 'Cascade'; 'Parallel'};

T = table(Structure, [0; E], [0; D], ...

'VariableNames', {'Structure','Maximum_Error','Deviation_percent'});

disp('Comparison Table:');

disp(T);

%% 6. Impulse Response Comparison

figure;

stem(n,y1,'filled'); hold on;

stem(n,y2,'--');

stem(n,yc,':','LineWidth',1.5);

stem(n,yp,'-.');

grid on;

xlabel('n');

ylabel('Amplitude');

title('Impulse Response of IIR Realizations');

legend('Direct Form-I','Direct Form-II','Cascade','Parallel');

%% 7. Error Plot

figure;

plot(n,abs(y1-y2),'--','LineWidth',1.2); hold on;

plot(n,abs(y1-yc),':','LineWidth',1.5);

plot(n,abs(y1-yp),'-.','LineWidth',1.2);

grid on;

xlabel('n');

ylabel('Absolute Error');

title('Numerical Error');

legend('DF-I vs DF-II','DF-I vs Cascade','DF-I vs Parallel');

%% 8. Frequency Response

figure;

freqz(b,a,1024);

title('Frequency Response of IIR Filter');

%% 9. Pole-Zero Plot

figure;

zplane(b,a);

title('Pole-Zero Plot');

# MATLAB Output Screenshots

Figure 1: Impulse response comparison of Direct Form-I, Direct Form-II, cascade and parallel realizations.

Figure 2: Reference impulse response of the IIR filter.

Figure 3: Numerical error between the filter realizations.

Figure 4: Frequency response of the IIR filter.

Figure 5: Pole-zero plot of the IIR filter.

# Output

The IIR filter was represented using Direct Form-I, Direct Form-II, cascade and parallel structures. The impulse responses, numerical errors, frequency response and pole-zero plot were obtained in MATLAB.

# Done By

Lokesh A (192512293)

B.E. Electronics and Communication Engineering (ECE)