DIGITAL SIGNAL PROCESSING LABORATORY

EXPERIMENT NO. 3

Experiment Name: Design of IIR Filters Using Impulse Invariance and Bilinear Transformation Methods Using MATLAB

Date of Experiment: 12 October 2026

# Objectives

To design digital IIR filters using the impulse invariance method.

To design digital IIR filters using the bilinear transformation method with prewarping.

To calculate and display the digital filter coefficients for both methods.

To compare the magnitude and phase responses obtained using the two methods.

# Program Code

clear;

clc;

close all;

Fs = 5000;

T = 1/Fs;

fc = 1000;

Omega_c = 2 * pi * fc;

% Analog Filter Prototype: H(s) = Omega_c / (s + Omega_c)

b_a = [Omega_c];

a_a = [1, Omega_c];

% 1. Impulse Invariance Method

[b_imp, a_imp] = impinvar(b_a, a_a, Fs);

% 2. Bilinear Transformation Method (with Prewarping)

Omega_p = (2/T) * tan(Omega_c * T / 2); % Prewarped frequency

b_a_pre = [Omega_p];

a_a_pre = [1, Omega_p];

[b_blt, a_blt] = bilinear(b_a_pre, a_a_pre, Fs);

% Display numerical outputs in Command Window

fprintf('=== EXPERIMENT 3: IMPULSE INVARIANCE vs BILINEAR TRANSFORMATION ===\n\n');

fprintf('Analog Cutoff Frequency (Omega_c): %.2f rad/s\n', Omega_c);

fprintf('Prewarped Analog Frequency (Omega_p): %.2f rad/s\n\n', Omega_p);

fprintf('--- IMPULSE INVARIANCE COEFFICIENTS ---\n');

fprintf('Digital Numerator (b_imp):\n');

disp(b_imp);

fprintf('Digital Denominator (a_imp):\n');

disp(a_imp);

fprintf('--- BILINEAR TRANSFORMATION COEFFICIENTS ---\n');

fprintf('Digital Numerator (b_blt):\n');

disp(b_blt);

fprintf('Digital Denominator (a_blt):\n');

disp(a_blt);

% Frequency responses

[H_imp, f] = freqz(b_imp, a_imp, 1024, Fs);

[H_blt, ~] = freqz(b_blt, a_blt, 1024, Fs);

% Plotting

figure('Name', 'Impulse Invariance vs Bilinear Transformation');

subplot(2,1,1);

plot(f, abs(H_imp), 'b', 'LineWidth', 1.5);

hold on;

plot(f, abs(H_blt), 'r--', 'LineWidth', 1.5);

grid on;

legend('Impulse Invariance', 'Bilinear Transformation (Prewarped)');

title('Magnitude Response Comparison');

xlabel('Frequency (Hz)');

ylabel('Absolute Magnitude');

subplot(2,1,2);

plot(f, angle(H_imp)*180/pi, 'b', 'LineWidth', 1.5);

hold on;

plot(f, angle(H_blt)*180/pi, 'r--', 'LineWidth', 1.5);

grid on;

legend('Impulse Invariance', 'Bilinear Transformation');

title('Phase Response Comparison');

xlabel('Frequency (Hz)');

ylabel('Phase (degrees)');

# MATLAB Output Screenshots

Figure 1: Magnitude and phase response comparison of impulse invariance and bilinear transformation.

Figure 2: MATLAB Online workspace showing the program and generated response plots.

Done by

Lokesh A

Register Number: 192512293

B.E. Electronics and Communication Engineering (ECE)