DIGITAL SIGNAL PROCESSING LABORATORY

EXPERIMENT NO. 8

Realization of FIR Filters Using Direct, Cascade and Linear-Phase Structures Using MATLAB

Date of Experiment: 12 October 2026

# Objectives

To realize an FIR filter using direct and cascade structures.

To verify the symmetry of FIR filter coefficients and its linear-phase property.

To compare the impulse responses obtained from direct and cascade realizations.

To analyze the magnitude response, phase response, group delay and numerical error.

# Program Code

clc;

clear;

close all;

%% FIR FILTER PARAMETERS

b = [0.0625 0.2500 0.3750 0.2500 0.0625];

Fs = 1000;

N = 128;

x = [1 zeros(1,N-1)];

n = 0:N-1;

%% 1. DIRECT REALIZATION

yd = filter(b,1,x);

%% 2. CASCADE REALIZATION

% H(z) = (0.25 + 0.5z^-1 + 0.25z^-2)^2

b1 = [0.25 0.5 0.25];

b2 = [0.25 0.5 0.25];

yc = filter(b1,1,x);

yc = filter(b2,1,yc);

%% 3. SYMMETRY AND LINEAR PHASE

symErr = max(abs(b-fliplr(b)));

D = (length(b)-1)/2;

if symErr < 1e-12

symmetry = 'Symmetric';

else

symmetry = 'Not Symmetric';

end

%% 4. FREQUENCY, PHASE AND GROUP DELAY

[H,w] = freqz(b,1,1024);

phase = unwrap(angle(H));

gd = grpdelay(b,1,1024);

%% 5. NUMERICAL COMPARISON

err = max(abs(yd-yc));

dev = err/max(abs(yd))*100;

%% 6. DISPLAY RESULTS

fprintf('\\n============================================\\n');

fprintf(' FIR FILTER REALIZATION RESULTS\\n');

fprintf('============================================\\n');

fprintf('\\nFIR Coefficients:\\n');

disp(b);

fprintf('Sampling Frequency = %d Hz\\n',Fs);

fprintf('Number of Samples = %d\\n',N);

fprintf('Coefficient Symmetry = %s\\n',symmetry);

fprintf('Symmetry Error = %.3e\\n',symErr);

fprintf('Group Delay = %.2f samples\\n',D);

fprintf('Maximum Error = %.3e\\n',err);

fprintf('Percentage Deviation = %.3e %%\\n',dev);

%% 7. FIRST 10 IMPULSE RESPONSE VALUES

fprintf('\\n============================================\\n');

fprintf('FIRST 10 IMPULSE RESPONSE VALUES\\n');

fprintf('============================================\\n');

fprintf('%4s %12s %12s\\n','n','Direct','Cascade');

for k = 1:10

fprintf('%4d %12.6f %12.6f\\n',n(k),yd(k),yc(k));

end

%% 8. CASCADE REALIZATION ZEROS

fprintf('\\n============================================\\n');

fprintf('CASCADE REALIZATION ZEROS\\n');

fprintf('============================================\\n');

z = roots(b);

disp(z);

%% 9. OUTPUT PARAMETER TABLE

Parameter = {

'Direct realization output'

'Cascade realization output'

'Impulse response'

'Phase response'

'Comparison of realizations'

};

Observation = {

'Direct FIR filtering'

'Cascade of second-order sections'

'Equal to FIR coefficient sequence'

'Linear phase'

'Equivalent transfer-function behavior'

};

Value = {

sprintf('[%.4f %.4f %.4f %.4f %.4f]',yd(1:5))

sprintf('[%.4f %.4f %.4f %.4f %.4f]',yc(1:5))

sprintf('[%.4f %.4f %.4f %.4f %.4f]',yd(1:5))

sprintf('Group delay = %.2f samples',D)

sprintf('Max error = %.3e; Deviation = %.3e %%',err,dev)

};

Unit_Note = {

'Amplitude'

'Amplitude'

'h[n]'

'Samples'

'Numerical precision'

};

Output_Table = table(Parameter,Observation,Value,Unit_Note);

fprintf('\\n============================================\\n');

fprintf('OUTPUT PARAMETER TABLE\\n');

fprintf('============================================\\n');

disp(Output_Table);

%% 10. DIRECT VS CASCADE IMPULSE RESPONSE

figure;

stem(n,yd,'filled');

hold on;

stem(n,yc,'--o','MarkerSize',3);

grid on;

xlabel('Sample n');

ylabel('Amplitude');

title('Direct and Cascade FIR Impulse Responses');

legend('Direct Realization','Cascade Realization');

%% 11. MAGNITUDE RESPONSE

figure;

plot(w/pi,abs(H),'LineWidth',1.5);

grid on;

xlabel('Normalized Frequency (x pi rad/sample)');

ylabel('|H(e^{j\omega})|');

title('FIR Magnitude Response');

xlim([0 1]);

%% 12. UNWRAPPED PHASE RESPONSE

figure;

plot(w/pi,phase,'LineWidth',1.5);

grid on;

xlabel('Normalized Frequency (x pi rad/sample)');

ylabel('Phase (radians)');

title('Unwrapped Phase Response');

xlim([0 1]);

%% 13. GROUP DELAY

figure;

plot(w/pi,gd,'LineWidth',1.5);

grid on;

xlabel('Normalized Frequency (x pi rad/sample)');

ylabel('Group Delay (samples)');

title('Group Delay');

xlim([0 1]);

ylim([0 3]);

%% 14. NUMERICAL ERROR

figure;

plot(n,abs(yd-yc),'LineWidth',1.5);

grid on;

xlabel('Sample n');

ylabel('Absolute Error');

title('Direct vs Cascade Numerical Error');

xlim([0 N-1]);

%% 15. FINAL RESULT

fprintf('\\n============================================\\n');

fprintf('FINAL RESULT\\n');

fprintf('============================================\\n');

fprintf('1. FIR filter realized using Direct structure.\\n');

fprintf('2. FIR filter realized using Cascade structure.\\n');

fprintf('3. Coefficients are %s.\\n',symmetry);

fprintf('4. Linear-phase group delay = %.2f samples.\\n',D);

fprintf('5. Maximum realization error = %.3e.\\n',err);

fprintf('6. Percentage deviation = %.3e %%\\n',dev);

fprintf('7. Direct and Cascade responses are equivalent.\\n');

# MATLAB Output Screenshots

Figure 1: MATLAB output for FIR filter realization.

Figure 2: MATLAB output for FIR filter realization.

# Done By

Lokesh A (192512293)

B.E. Electronics and Communication Engineering (ECE)